README – Rider Telemetry System (ESP32 + IMU + Local Web Dashboard)

Overview

This project is a wearable rider telemetry system using an ESP32 and MPU6050 IMU. It measures rider motion, braking behaviour, and movement state in real time, and displays the data on a local web dashboard. All data can be downloaded as a CSV file for analysis. It runs fully offline with no cloud required. GPS support is included safely, but the system continues to work even if GPS doesn’t lock.

Hardware Needed

- ESP32 development board

- MPU6050 IMU module

- (Optional) NEO-6M GPS module

- USB cable or 5V power bank

- Jumper wires

Basic wiring:
MPU6050 → SDA to GPIO 21, SCL to GPIO 22, VCC to 3.3V, GND to GND
GPS (optional) → TX to GPIO16, RX to GPIO17, VCC to 5V, GND to GND

Software Requirements

- Arduino IDE installed

- ESP32 board package installed in Arduino IDE

- Library required: TinyGPSPlus (only if using GPS)

Setup Instructions

- Open the code in Arduino IDE.

- Install the required library if using GPS.

- (VERY IMP) Update the Wi-Fi hotspot name and password in the code.

- Upload the code to the ESP32.

- Turn on your phone hotspot and keep your phone connected.

- Open the Serial Monitor to see the IP address printed by the ESP32.

- Enter that IP address in a browser to open the dashboard.

Example browser link format:
http://<IP shown in Serial Monitor>/ (most probably – 10.117.126.199 )

How It Works

- Connect ESP32 to your phone hotspot.

- Access the live dashboard in any browser.

- Pitch, roll, filtered acceleration, linear acceleration, state detection, and braking status update live.

- You can view a results summary page and download the recorded data as a CSV file.

If GPS is connected and gets a satellite lock outdoors, speed and location will also appear. If not, it safely shows “Searching for GPS…” and logs zeros instead, without affecting IMU functions.

Customization for Other Users

- Change Wi-Fi hotspot name and password at the top of the code.

- Edit the logging interval (default 500 ms) to store data more or less frequently.

- Increase or reduce sample storage size if needed.

- GPS can be disabled by removing the GPS-related lines; the code still works fully with IMU only.

Important Notes

- GPS only works outdoors and may take 30–90 seconds for a satellite fix.

- The system works completely without GPS, so it remains stable in all conditions.

- CSV download contains all recorded sensor values and timestamps for analysis.